; GPRS Modems Configuration File

; Manufacturer: Alice Systems

;

; Copyright (c) 2001 Wireless Login AB

; Version = 120 APN

;---------------------------------------------------------------------

; 26/06/2002 H-P TRINH		120 Add support for Nokia 7650, D211, // R600, Option, Novatel, Sierra wireless

; 15/05/2002 H-P TRINH		119 Modify inf name for S45, ME45, Gd76, GD96, V60, V66

; 11/02/2002 G. Ensuque		118 Added entry for 'T68' (reply to +GMM on WinME)

; 14/01/2002 H-P TRINH		117 Add OPTION GPRS Globe Trotter card

; 09/01/2002 J.Kastensson	116 Added InfFile key to T39/S45/8310

; 07/12/2001 S.Awad		115 added HSCSD support for Ericsson T68

; 07/12/2001 S.Awad		115 added HSCSD support for Nokia Card Phone 2.0

; 27/11/2001 G.Ensuque		114 added NEC FOMA card

; 20/11/2001 J.Kastensson	Added HSCSDOptional command

; 07/11/2001 J.Kastensson	Added HSCSD-commmnds for Ericsson BVR520m/Nokia 8310/Default

; 09/10/2001 M. Linde		112 Added Ericsson T68 + Attach T65

; 02/10/2001 I.  Biffi      		111 Added Novatel  - (Martin / Microcell)

; 21/09/2001 L. Sj�berg        	Added Motorola V60

; 19/09/2001 L. Axelsson 	Added Ericsson T65

; 10/09/2001 I   Biffi		106 Modified Siemens S45 + ME45

; 10/09/2001 I   Biffi		106 Modified CID for T39

; 05/09/2001 L. Sj�berg		Added Nokia 8310

; 04/09/2001 L. Sj�berg		modifed default behaviour to short PDP command.

; 30/08/2001 L. Axelsson	Added Siemens 3618

; 15/08/2001 S. Awad		Added Siemens ME45

; 14/08/2001 L. Axelsson	Added Siemens 6618

; 14/08/2001 L. Axelsson	Added Siemens S45

; 09/08/2001 L. Sj�berg		Corrected CID to #4 for T39 on PockectPC

; 17/07/2001 H-P Trinh		Added Ericsson T39, Motorola V66 and Timeport 280

; 28/06/2001 H-P Trinh		Added Samsung SGH-Q100

; 28/06/2001 G. Ensuque		added Panasonic GD93P

; 25/06/2001 H-P Trinh		add modem inf files parameters

; 21/06/2001 G. Ensuque		Added Attach parameter, changed cid to 6 for E///,

; 				Removed cid from (default) phonenumber

; 11/05/2001 L. Axelsson	Added Ericsson handset and corrected QoS for Mitsubishi

; 09/05/2001 L. Axelsson	Corrected QoS settings for Sagem and Motorola handset

; 07/03/2001 H-P. Trinh       	Corrected Sagem Settings

; 06/02/2001 H-P. Trinh		Added SAGEM M9XX Series settings

; 15/01/2001 G. Ensuque		Added (Default) behaviour

; 11/01/2001 H-P. Trinh		Added Ericsson T520m +cgmi

; 12/12/2000 G. Ensuque		Name changed to GPRSmdms.cfg

: 29/11/2000 G. Ensuque		Added SAGEM 900 Series settings

; 26/11/2000 G. Ensuque		Added PhoneNumber parameter

; ---------------------------------------------------------------------

;***ERICSSON***

[ERICSSON  :1130202-BVT68]

DefinePdpContext=AT+CGDCONT=10,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=10,0,0,0,0,0;+CGQMIN=10,0,0,0,0,0

PhoneNumber=*99***10#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=erit68.inf

[ERICSSON  :1130201-BVT68]

DefinePdpContext=AT+CGDCONT=10,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=10,0,0,0,0,0;+CGQMIN=10,0,0,0,0,0

PhoneNumber=*99***10#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=erit68.inf

[R600:102001-BVR600]

DefinePdpContext=AT+CGDCONT=5,"%TYPE","%APN",%ADDR,%DCOMP,%HCOMP

Optional=AT+CGQREQ=5,0,0,0,0,0;+CGQMIN=5,0,0,0,0,0

PhoneNumber=*99***5#

Attach=AT+CGATT=1

InfFile=EriR600.inf

[ERICSSON  :T68]

DefinePdpContext=AT+CGDCONT=10,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=10,0,0,0,0,0;+CGQMIN=10,0,0,0,0,0

PhoneNumber=*99***10#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=erit68.inf

[ERICSSON  :1101901-BVT65]

DefinePdpContext=AT+CGDCONT=4,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=4,0,0,0,0,0;+CGQMIN=4,0,0,0,0,0

PhoneNumber=*99***4#

Attach=AT+CGATT=1

InfFile=erit65.inf

[ERICSSON  :1130102-BVT39m]

DefinePdpContext=AT+CGDCONT=4,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=4,0,0,0,0,0;+CGQMIN=4,0,0,0,0,0

PhoneNumber=*99***4#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=erit39.inf

[ERICSSON  :1130101-CNR520mc]

DefinePdpContext=AT+CGDCONT=6,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=6,0,0,0,0,0;+CGQMIN=6,0,0,0,0,0

PhoneNumber=*99***6#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=erir520.inf

[ERICSSON  :1130101-BVR520m]

DefinePdpContext=AT+CGDCONT=6,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=6,0,0,0,0,0;+CGQMIN=6,0,0,0,0,0

PhoneNumber=*99***6#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=erir520.inf

[ERICSSON:1101101-BVR520m]

DefinePdpContext=AT+CGDCONT=6,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=6,0,0,0,0,0;+CGQMIN=6,0,0,0,0,0

PhoneNumber=*99***6#

Attach=AT+CGATT=1

InfFile=erir520.inf

[ERICSSON  :1101101-BVR520m]

DefinePdpContext=AT+CGDCONT=6,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=6,0,0,0,0,0;+CGQMIN=6,0,0,0,0,0

PhoneNumber=*99***6#

Attach=AT+CGATT=1

InfFile=erir520.inf

; ***MITSUBISHI***

[Mitsubishi:MT-Data-Outside]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%XADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=Mitsubishi Geo.INF

[Mitsubishi:M5]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%XADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=Trium Eclipse modem.inf

; ***MOTOROLA***

[+CGMI: "Motorola CE, Copyright 2000":+CGMM: "GSM900","GSM1800","GSM1900","MODEL=V.series 66"]

DefinePdpContext=AT+CGDCONT=1,%TYPE,%APN,%ADDR,%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99#

Attach=AT+CGATT=1

InfFile=Motorola V66.inf

[+CGMI: "Motorola CE, Copyright 2000":+CGMM: "GSM900","GSM1800","GSM1900","MODEL=280"]

DefinePdpContext=AT+CGDCONT=1,%TYPE,%APN,%ADDR,%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99#

Attach=AT+CGATT=1

InfFile=Motorola T280.inf

[+CGMI: "Motorola CE, Copyright 2000":+CGMM: "GSM900","GSM1800","GSM1900","MODEL=V60"]

DefinePdpContext=AT+CGDCONT=3,%TYPE,%APN,%ADDR,%DCOMP,%HCOMP

Optional=AT+CGQREQ=3,0,0,0,0,0;+CGQMIN=3,0,0,0,0,0

PhoneNumber=*99***3#

Attach=AT+CGATT=1

InfFile=Motorola V60.inf

[+CGMI: "Motorola CE, Copyright 2000":+CGMM: "GSM900","GSM1800","MODEL=V70"]

DefinePdpContext=AT+CGDCONT=1,%TYPE,%APN,%ADDR,%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=Motorola V70.inf

[+CGMI: "Motorola":+CGMM: "L Series"]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99#

Attach=AT+CGATT=1

InfFile=Motorola T260_A008.inf

[+CGMI: "Motorola":+CGMM: "TaiChi Series"]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=Motorola TaiChi.inf

; ***NEC***

[NEC Corporation:FOMA Mobile Card N2]

DefinePdpContext=AT

Optional=AT

PhoneNumber=*99***1#

Attach=AT

infFile=DcmDCN02.inf

[NEC Corporation:NEC DB7000]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN"

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=

; ***NOKIA***

[Nokia Mobile Phones:Nokia 8310]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=2;+CGQMIN=2

PhoneNumber=*99***2#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,0

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=AS Nokia 8310 modem.inf

[Nokia Mobile Phones:Nokia 6310]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=2;+CGQMIN=2

PhoneNumber=*99***2#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,0

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=AS Nokia 6310 modem.inf

[Nokia Mobile Phones:Nokia 6510]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=2;+CGQMIN=2

PhoneNumber=*99***2#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,0

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=AS Nokia 6510 modem.inf

[Nokia Mobile Phones:Nokia 6210]

RasInit=&S0

HSCSDspeed=AT+CHSN=%NUM,0,0,0

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=AS Nokia 6210 modem.inf

[Nokia Mobile Phones:Nokia Card Phone RPM-1 GSM900/1800]

HSCSDspeed=AT+CHSN=%NUM,0,0,0

HSCSDmode=AT+CBST=%MODE,0,1

;Nokia 7110

[Nokia Mobile Phones:NSE-5]

RasInit=&S0

[Nokia:Nokia 7650]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1;+CGQMIN=1

PhoneNumber=*99***1#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,0

HSCSDmode=AT+CBST=%MODE,0,1

InfFile=AS Nokia 7650 modem.inf

[Nokia Corporation:Nokia D211]

Init=CheckPin,CheckNetworkReg,CheckGprsNetworkReg,ATE0,ATV1

CheckPIN=AT+CPIN?

SetPIN=AT+CPIN=%PIN

CheckIMSI=AT+CIMI

NetworkRegSet=AT+CREG=0

NetworkRegCheck=AT+CREG?

NetworkRegRetryDelay=2000

NetworkRegRetryCount=5

SetSelectOp=AT+COPS=0,2

CheckSelectOp=AT+COPS?

GPRSNetworkRegSet=AT+CGREG=0

GPRSNetworkRegCheck=AT+CGREG?

GPRSNetworkRegRetryDelay=1000

GPRSNetworkRegRetryCount=10

CheckAttach=AT+CGATT?

Attach=AT+CGATT=1

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

HangUp=ATH

SetEventReport=AT+CEER?

; ***NOVATEL***

[Novatel Wireless Inc.:GPRS PC Card 56600]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=2;+CGQMIN=2

PhoneNumber=*99***2#

Attach=AT+CGATT=1

[Novatel Wireless, Inc.:GPRS PC Card 900/1800]

Init=CheckPin,CheckNetworkReg,CheckGprsNetworkReg,ATE0,ATV1

CheckPIN=AT+CPIN?

CheckPIN.Precomdelay=1500

CheckPIN.Readdelay=1000

CheckPIN.Postcomdelay=500

CheckPIN.RetryCount=5

SetPIN=AT+CPIN=%PIN

SetPIN.Postcomdelay=3000

CheckIMSI=AT+CIMI

NetworkRegSet=AT+CREG=0

NetworkRegCheck=AT+CREG?

NetworkRegRetryDelay=4000

NetworkRegRetryCount=10

SetSelectOp=AT+COPS=0,2

CheckSelectOp=AT+COPS?

GPRSNetworkRegSet=AT+CGREG=0

GPRSNetworkRegCheck=AT+CGREG?

GPRSNetworkRegRetryDelay=7000

GPRSNetworkRegRetryCount=10

CheckAttach=AT+CGATT?

Attach=AT+CGATT=1

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

;Optional=AT+CGQREQ=1;+CGQMIN=1

;Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

HangUp=ATH

SetEventReport=AT+CEER?

InfFile=mdmgl01.inf

; ***OPTION***

[Option Wireless Technologies:Globetrotter GPRS PCMCIA]

DefinePdpContext=AT+CGDCONT=4,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT

PhoneNumber=*99***4#

Attach=AT+CGATT=1

InfFile=Globetr.INF

; ***PANASONIC***

[Panasonic:GD76]

DefinePdpContext=AT+CGDCONT=4,"%TYPE","%APN","%ADDR"

Optional=AT+CGQREQ=4;+CGQMIN=4

PhoneNumber=*99***4#

Attach=AT+CGATT=1

InfFile=AS Panasonic GD76 modem.inf

[Panasonic:GD93P]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;AT+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=AS Panasonic modem.inf

[Panasonic:GD95]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR"

Optional=AT+CGQREQ=1,,,3,,;+CGQMIN=1,,,3,,

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=AS Panasonic modem.inf

[Panasonic:GD96]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR"

Optional=AT+CGQREQ=1,,,3,,;+CGQMIN=1,,,3,,

PhoneNumber=*99***1#

Attach=AT+CGATT=1

InfFile=AS Panasonic GD96 modem.inf

; ***SAGEM***

[+CGMI: SAGEM :+CGMM: M9XX GPRS]

DefinePdpContext=AT&d2;+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0

PhoneNumber=*99*1#

Attach=AT+CGATT=1

InfFile=AS Sagem modem.inf

; ***SAMSUNG***

[Samsung:SGH-Q100]

DefinePdpContext=AT&k3;AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;AT+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99*1,1#

Attach=AT+CGATT=1

InfFile=AS Samsung modem.inf

; ***SIEMENS***

[SIEMENS:S45]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN"

Optional=AT+CGQREQ=2,0,0,0,0,0;+CGQMIN=2,0,0,0,0,0

PhoneNumber=*99***2#

Attach=AT+CGATT=1

InfFile=Siemens S45 modem.inf

[SIEMENS:ME45]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN"

Optional=AT+CGQREQ=2,0,0,0,0,0;+CGQMIN=2,0,0,0,0,0

PhoneNumber=*99***2#

Attach=AT+CGATT=1

InfFile=Siemens Me45 modem.inf

[SIEMENS:6618]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN"

Optional=AT+CGQREQ=2,0,0,0,0,0;+CGQMIN=2,0,0,0,0,0

PhoneNumber=*99***2#

Attach=AT+CGATT=1

InfFile=Siemens modem.inf

[SIEMENS:3618]

DefinePdpContext=AT+CGDCONT=2,"%TYPE","%APN"

Optional=AT+CGQREQ=2,0,0,0,0,0;+CGQMIN=2,0,0,0,0,0

PhoneNumber=*99***2#

Attach=AT+CGATT=1

InfFile=Siemens modem.inf

; ***SIERRA WIRELESS***

[Sierra Wireless Inc.:A7XX]

Init=CheckPin,CheckNetworkReg,CheckGprsNetworkReg,ATE0,ATV1

CheckPIN=AT+CPIN?

SetPIN=AT+CPIN=%PIN

CheckIMSI=AT+CIMI

NetworkRegSet=AT+CREG=0

NetworkRegCheck=AT+CREG?

NetworkRegRetryDelay=2000

NetworkRegRetryCount=5

SetSelectOp=AT+COPS=0,2

CheckSelectOp=AT+COPS?

GPRSNetworkRegSet=AT+CGREG=0

GPRSNetworkRegCheck=AT+CGREG?

GPRSNetworkRegRetryDelay=1000

GPRSNetworkRegRetryCount=10

CheckAttach=AT+CGATT?

Attach=AT+CGATT=1

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN","%ADDR",%DCOMP,%HCOMP

Optional=AT+CGQREQ=1,0,0,0,0,0;+CGQMIN=1,0,0,0,0,0

PhoneNumber=*99***1#

HangUp=ATH

SetEventReport=AT+CEER?

[Default]

DefinePdpContext=AT+CGDCONT=1,"%TYPE","%APN"

Optional=AT+CGQREQ=1;+CGQMIN=1

PhoneNumber=*99***1#

Attach=AT+CGATT=1

HSCSDspeed=AT+CHSN=%NUM,0,0,12

HSCSDmode=AT+CBST=%MODE,0,1

HSCSDOptional=AT